# IMXRT1176 FreeRTOS Bring-UP

- Open MCUXpresso IDE to create a new C++ project for the i.MX RT1176 EVK board, opting for minimal configuration settings during the setup wizard while ensuring that RTOS is selected under the Operating System options, then compile the project and flash it to run on the hardware.

- Integrate features like USB, FatFS, LittleFS, flash, and OSA into the MCUXpresso IDE project configuration alongside the RTOS.

- Import the video and VG-Lite rendering code from an existing SDK reference project into your configured workspace to enable hardware-accelerated graphics and video playback.

- Make changes to FreeRTOSConfig.h

  - configTOTAL_HEAP_SIZE: Define the total available heap memory space for task creation, queues, and resources, ensuring it accommodates VG-Lite and video buffer requirements.

  - configMAX_PRIORITIES: Set the maximum number of task priorities available in the scheduler.

  - configQUEUE_REGISTRY_SIZE: Configure the maximum number of queues and semaphores that can be registered for debugging tools.

  - configFRTOS_MEMORY_SCHEME: Select the memory allocation scheme implementation (such as heap_4).

  - configINCLUDE_FREERTOS_TASK_C_ADDITIONS_H: Enable the inclusion of the task additions header if required by the SDK framework.

- Update the project's memory map in the linker script to designate SDRAM as the default RAM region instead of the limited 256 KB SRAM_DTC_CM7, preventing memory overflows caused by large assets like VG-Lite framebuffers and video buffers. A sample linker script is as follows.

  - BOARD_FLASH (rx) : ORIGIN = 0x30000000, LENGTH = 0x1000000 /* 16M bytes (alias Flash) */

  - BOARD_SDRAM (rwx) : ORIGIN = 0x80000000, LENGTH = 0x1a00000 /* 26M bytes (alias RAM) */

  - NCACHE_REGION (rwx) : ORIGIN = 0x83800000, LENGTH = 0x800000 /* 8M bytes (alias RAM2) */

  - SRAM_DTC_cm7 (rwx) : ORIGIN = 0x20000000, LENGTH = 0x40000 /* 256K bytes (alias RAM3) */

  - SRAM_ITC_cm7 (rwx) : ORIGIN = 0x0, LENGTH = 0x40000 /* 256K bytes (alias RAM4) */

  - SRAM_OC1 (rwx) : ORIGIN = 0x20240000, LENGTH = 0x80000 /* 512K bytes (alias RAM5) */

  - SRAM_OC2 (rwx) : ORIGIN = 0x202c0000, LENGTH = 0x80000 /* 512K bytes (alias RAM6) */

  - SRAM_OC_ECC1 (rwx) : ORIGIN = 0x20340000, LENGTH = 0x10000 /* 64K bytes (alias RAM7) */

  - SRAM_OC_ECC2 (rwx) : ORIGIN = 0x20350000, LENGTH = 0x10000 /* 64K bytes (alias RAM8) */

  - SRAM_OC_cm7 (rwx) : ORIGIN = 0x20360000, LENGTH = 0x20000 /* 128K bytes (alias RAM9) */

- Modify pinmux.c (or use the MCUXpresso Pins Tool) to map the specific peripheral pins for both the display and touch controller according to the i.MX RT1176 EVK hardware layout:

  - Display Interface: Route the parallel LCD or MIPI-DSI data lanes, clock, and synchronization signals.

  - Touch Controller: Configure the designated LPI2C or SPI communication pins along with the touch interrupt and reset GPIO lines.

- Enable the following preprocessor macros in your project settings to configure the target device, memory map, graphics framework, and FreeRTOS integration:

  - Target & Core: CPU_MIMXRT1176DVMAA, CPU_MIMXRT1176DVMAA_cm7, __MCUXPRESSO, __USE_CMSIS, __NEWLIB__, DEBUG

  - Memory & Boot: USE_SDRAM, XIP_EXTERNAL_FLASH=1, XIP_BOOT_HEADER_ENABLE=1, XIP_BOOT_HEADER_DCD_ENABLE=1, DATA_SECTION_IS_CACHEABLE=1

  - OS & System Drivers: RTOS, SDK_OS_FREE_RTOS, FSL_SDK_DRIVER_QUICK_ACCESS_ENABLE=1, MCUXPRESSO_SDK, SDK_I2C_BASED_COMPONENT_USED, SDK_DEBUGCONSOLE=1, SERIAL_PORT_TYPE_UART=1, PRINTF_FLOAT_ENABLE=0

  - Embedded Wizard & VG-Lite: VG_BLIT_WORKAROUND=1, EW_USE_VGLITE_INDEX8=0, EW_USE_FREE_RTOS=1, EW_FRAME_BUFFER_COLOR_FORMAT=EW_FRAME_BUFFER_COLOR_FORMAT_RGBA8888, EW_SURFACE_ROTATION=90

- Integrate the Embedded Wizard source code and libraries into your project workspace.

  - Copy Embedded Wizard Files: Transfer the Embedded Wizard generated code (GeneratedCode), platform package runtime sources, and header files into your project's directory structure (typically under source or dedicated subfolders).

  - Integrate VG-Lite Platform Files: Include the Embedded Wizard VG-Lite display driver integration files (such as ew_bsp_display.c and ew_bsp_touch.c configured for VG-Lite) so that the UI engine uses the hardware accelerator.

  - Memory management for Embedded Wizard: Configure Embedded Wizard's memory management within ewconfig.h by defining the EW_MEMORY_POOL_ADDR and EW_MEMORY_POOL_SIZE parameters to allocate its graphics resource pool within SDRAM.

  - Update Include Paths and Symbols: Configure MCUXpresso IDE under C/C++ Build > Settings > MCU C/C++ Compiler > Includes and Preprocessor to point to the new Embedded Wizard and VG-Lite header directories.

  - Add Libraries to Linker: Ensure the required VG-Lite graphics libraries and precompiled Embedded Wizard archives are linked correctly under MCU C/C++ Linker > Libraries.

  - Initialize the UI Task: Create a dedicated FreeRTOS task in your application that calls the Embedded Wizard initialization routine (EwInit()), sets up the frame buffers in SDRAM, and executes the main update loop alongside touch event handling